| Показатель | ПАВ-6 | АН-8-ПАВ |
| --- | --- | --- |
| Ширина захвата, м | 6,0 | 8,0 |
| Требуемая мощность колесного трактора, л. с. | от 150 | от 250 |
| Тип соединения с трактором | полунавесной Cat. III | полуприцепной, палец 50 мм |
| Требуемое количество пар гидравлических выводов | 2 | 3 |
| Глубина обработки почвы, см | до 8 |  |
| Рабочая скорость, км/ч | 8-10 |  |
| Количество рабочих секций, шт. | 4 |  |
| Количество рабочих зон, шт. | 6 |  |
| Диаметр опорных катков, мм | 270 или 330 |  |
| Ширина стрельчатой лапы, мм | 270 |  |
| Тип защиты стойки | срезной болт |  |
| Расстояние между стойками, мм | 250 |  |
| Диаметр кольчато-шпорового катка, мм | 400 |  |
| Шины | 15.5/65-18 | 520/50-17 |
| Длина в транспортном положении, м | 5,65 | 6,05 |
| Ширина в транспортном положении, м | 3,0 |  |
| Высота в транспортном положении, м | 2,39 | 4,2 |
| Масса, кг | 4500 | 7220 |

| Parameters | PAV-6 | AN-8-PAV |
| --- | --- | --- |
| Working width, m | 6,0 | 8,0 |
| Tractor requirement, hp | from 150 | from 250 |
| Connection | semi-mounted, Cat. III | semi-trailed, pin 50 mm |
| Required number of paired hydraulic outlets | 2 | 3 |
| Working depth, cm | < 8 |  |
| Working speed, km/h | 8-10 |  |
| Number of working sections, pcs | 4 |  |
| Number of working zones, pcs | 6 |  |
| Roller diameter, mm | 270 or 330 |  |
| A-blade width, mm | 270 |  |
| Tine overload protection | shear bolt |  |
| Tine spacing, mm | 250 |  |
| Crosskill roller diameter, mm | 400 |  |
| Tires | 15.5/65-18 | 520/50-17 |
| Dimensions in transport position |  |  |
| Length, m | 5,65 | 6,05 |
| Width, m | 3,0 |  |
| Height, m | 2,39 | 4,2 |
| Weight, kg | 4500 | 7220 |