Main technical data: Wavelength range; 790..820 or 940..980nm. Spectral width (FWHM); 2 nm. Beam divergance (FWHM); 40x10 degrees. Temperature coefficients. threshold current, T°; 120..140°C. T° can be modeled as ITH2=ITH1exp[(T2-T1)/To]. operating current; 0.8%/°C. wavelength generation; 0.3nm/°C. Thermal resistance; 5..10°C/W. Series resistance, typical; Rs=(Voper-1,5V)/Iop. Monitor photodiode; sensitivity; 0.3..10 mkA/mW. operates without reverse bias. Thermoelectric Cooler. Max. Drive Current; TB-31-0,6/0,8 for ATC-C500; 2.0 A. TB-17-1,0/0,7 for ATC-C1000; 7.0 A. Max. Drive Voltage; TB-31-0,6/0,8 for ATC-C500; 3.5 V. TB-17-1,0/0,7 for ATC-C1000; 2.0 V. Thermistor R @ +25°C; 10kOhm ± 5%. Typical ATC-SD laser diodes and arrays characteristics Typical Light vs Current characteristics. Typical Emission spectrum; Typical Farfield energy distribution.

High power diode lasers are - according to International Electronical Commission Standard - class 4 laser products. The lEC-Standard includes safety regulations for eye and personnel protection that must be observed to avoid any harm to operating personnel.

Основные технические характеристики:

| Длина волны излучения | 790..820 или 940..980 нм |  |
| --- | --- | --- |
| Спектральная ширина (FWHM) | 2 нм |  |
| Расходимость излучения (FWHM) | 40x10 градусов |  |
| Температурные коэффициенты |  |  |
| Пороговый ток, T° |  | 120..140 °C |
|  |  | T° определяется выражением ITH2=ITH1exp[(T2-T1)/To] |
| Рабочий ток |  | 0.8 %/°C |
| Длина волны генерации |  | 0.3 нм/°C |
| Тепловое сопротивление |  | 3..8 °C/Вт |
| Последовательное омическое сопротивление, определяется выражением |  | Rs=(Voper-1,5 В)/Iop |
| Фотодиод обратной связи | чувствительность | 0.3..10 мкА/мВт |
|  | работает без обратного смещения |  |

| Термобатарея |  |  |
| --- | --- | --- |
| Максимальный ток | TB-31-0,6/0,8 для ATC-C500 | 2.0 A |
|  | TB-17-1,0/0,7 для ATC-C1000 | 7.0 A |
| Maксимальное напряжение | TB-31-0,6/0,8 для ATC-C500 | 3.5 В |
|  | TB-17-1,0/0,7 для ATC-C1000 | 2.0 В |
| Tерморезистор R при +25 °C | 10 КОм ± 5 % |  |

| Типичные ватт-амперные характеристики |  |  |
| --- | --- | --- |
| ATC-C1000-380 | ATC-10000-200 | ATC-C15000-400 |
|  |  | ATC-C8000-200 |
| ATC-Q250-10-810 | ATC-Q120-10-940 |  |

| Типичный спектр излучения | Типичное распределение энергии в дальнем поле |  |
| --- | --- | --- |