Спецификация

СЗМ методы

На воздухе и в жидкости:

АСМ (контактная + полуконтактная + бесконтактная) / Латерально-Силовая Микроскопия / Отображение Фазы/ Модуляция Силы / Отображение Адгезионных Сил / Литографии: АСМ (Силовая)

Только на воздухе:

СТМ / МСМ / ЭСМ / СЕМ / Метод Зонда Кельвина / Отображение Сопротивления Растекания / AFAM (по требованию)/Литографии: АСМ (Силовая, Токовая), СТМ

Технические характеристики

| Тип сканирования |  | Скани- рование образцом | Скани- рование зондом* |
| --- | --- | --- | --- |
| Размер образца |  | До 40 мм в диаметре, до 15 мм в высоту | До 100 мм в диаметре, до 15 мм в высоту |
| Вес образца |  | До 100 г | До 300 г |
| XY позиционирование образца |  | 5x5 мм |  |
| Разрешение позиционирования |  | разрешение - 5 мкм минимальное перемещение - 2 мкм |  |
| Поле сканирования |  | 100x100x10 мкм 3x3x2,6 мкм не более 1x1x1 мкм | 100x100x10 мкм 50x50x5 мкм |
|  |  | До 200x200x20 мкм**(метод DualScanTM) |  |
| Нелинейность, XY (с датчиками обратной связи) |  | ≤0.1% | ≤0.15% |
| Уровень шума, Z (СКВ в полосе 1000 Гц) | С датчиками | 0.04 нм (типично), 0.06 нм | 0.06 нм (типично), 0.07 нм |
|  | Без датчиков | 0.03 нм | 0.05 нм |
| Уровень шума, XY*** (СКВ в полосе 200 Гц) | С датчиками | 0.2 нм (типично), 0.3 нм (XY 100 мкм) | 0.1 нм (типично), 0.2 нм (XY 50 мкм) |
|  | Без датчиков | 0.02 нм (XY 100 мкм), 0.001 нм (XY 3 мкм) | 0.01 нм (XY 50 мкм), |
| Ошибка измерения линейных размеров (с датчиками) |  | ±0.5% | ±1.2% |
| Система видео- наблюдения | Оптическое разрешение | 1 мкм (0.4 мкм по требованию, NA 0.7)**** | 3 мкм |
|  | Поле зрения | 4.5-0.4 мм | 2.0-0.4 мм |
|  | Непре- рывный зум | возможно | возможно |
| Вибро- изоляция | Активная | 0.7-1000 Гц |  |
|  | Пассивная | выше 1 кГц |  |

* Сканирующая измерительная головка может быть использована в качестве выносной головки при измерениях образцов неограниченных размеров.
** По требованию заказчика поле сканирования может быть увеличено вплоть до 200x200x20 мкм.
*** Встроенные емкостные датчики обладают рекордно низким уровнем шумов, что позволяет осуществлять сканирование с замкнутой петлей обратной связи при размерах области сканирования до 50x50 нм.
**** По требованию заказчика возможно дополнительно установить оптическую систему с высоким разрешением.

Specifications

Measuring modes and techniques

In air and liquid:

AFM (contact + semi-contact + non-contact) / Lateral Force Microscopy / Phase Imaging/ Force Modulation / Adhesion Force Imaging / Lithography: AFM (Force)

In air only:

STM / Magnetic Force Microscopy / Electrostatic Force Microscopy / Scanning Capacitance Microscopy / Kelvin Probe Microscopy / Spreading Resistance Imaging / Lithography: AFM (Current), STM / AFAM (optional)

| Scan type |  | Scanning by sample | Scanning by probe* |
| --- | --- | --- | --- |
| Sample size |  | Up to 40 mm in diameter, to 15 mm in height | Up to 100 mm in diameter, up to 15 mm in height |
| Sample weight |  | Up to 100 g | Up to 300 g |
| XY sample positiniong |  | 5x5 mm |  |
| Positioning resolution |  | readable resolution - 5 um sensitivity - 2 um |  |
| Scan range |  | 100x100x10 um 3x3x2,6 um Less than 1x1x1 um | 100x100x10 um 50x50x5 um |
|  |  | Up to 200x200x20 um**(DualScanTM mode) |  |
| Non linearity, XY (with closed loop sensors) |  | ≤ 0.1% | ≤ 0.15% |
| Noise level, Z (RMS in bandwidth 1000 Hz) | With sensors | 0.04 nm (typically), ≤ 0.06 nm | 0.06 nm (typically), ≤ 0.07 nm |
|  | Without sensors | 0.03 nm | 0.05 nm |
| Noise level, XY*** (RMS in bandwidth 200 Hz) | With sensors | 0.2 nm (typically), ≤ 0.3 nm (XY 100 um) | 0.1 nm (typically), ≤ 0.2 nm (XY 50 um) |
|  | Without sensors | 0.02 nm (XY 100 um), 0.001 nm (XY 3 um) | 0.01 nm (XY 50 um), |
| Linear dimension estimation error (with sensors) |  | ± 0.5% | ± 1.2% |
| Optical viewing system | Optical resolution | 1 um (0.4 um optional, NA 0.7)**** | 3 um |
|  | Field of view | 4.5-0.4 mm | 2.0-0.4 mm |
|  | Continuous zoom | available | available |
| Vibration isolation | Active | 0.7-1000 Hz |  |
|  | Passive | above 1 kHz |  |

* Scanning head can be configured to serve as a stand-alone device for specimens of unlimited sizes.

** Optionally can be expanded to 200x200x20 мm.

*** Built-in capacitive sensors have extremely low noise and any area down to 50x50 nm can be scanned with closed-loop control.

**** High Resolution Viewing system (HRV head) is optional and provides additional functionality making it possible to generate and detect tip-localized aperture less near-field effects.